Assignment module 2 : A+ - Peripherals And Power Supply

- Which of the following precautions should be taken before working on computer hardware?

Ans. Wear an anti-static wrist strap to prevent damage from electrostatic discharge.

- What is the purpose of thermal paste during CPU installation?

Ans. To improve thermal conductivity between the CPU and the heat sink.

- Which tool is used to measure the output voltage of a power supply unit (PSU)?

Ans. Multimeter

- Which component is responsible for storing BIOS settings, such as date and time, even when the computer is powered off?

Ans. CMOS battery

- True or False: When installing a new hard drive, it is essential to format it before use.

Ans. True

- True or False: A POST (Power-On Self-Test) error indicates a problem with the CPU.

Ans. False

- True or False: It is safe to remove a USB flash drive from a computer without ejecting it first.

Ans. False

- Describe the steps involved in installing a new graphics card in a desktop computer.

Ans. 1. Power Off and Unplug

- Shut down the computer completely.

- Disconnect the power cable and all connected devices (monitor, keyboard, etc.).

- Press the power button for a few seconds to discharge any remaining electricity.

2. Open the Computer Case

- Remove the side panel of your desktop case using a screwdriver.

- Place the case on a flat, clean surface with good lighting.

3. Remove the Old Graphics Card (if present)

- Unscrew the retaining screws that hold the old card to the case.

- Gently unlock the PCIe slot latch at the end of the card.

- Carefully pull out the old graphics card straight from the slot.

4. Locate the PCI Express x16 Slot

- Identify the main PCIe x16 slot (usually the longest slot near the CPU).

- Make sure the slot and area are clear of dust or cables.

5. Insert the New Graphics Card

- Line up the card’s connector with the PCIe slot.

- Push it down firmly and evenly until it clicks into place.

- Screw the card’s bracket into the case for stability.

6. Connect Power Cables

- Many modern GPUs require extra PCIe power connectors (6-pin or 8-pin).

- Plug the power supply cables into the card securely.

7. Close the Case and Reconnect

- Replace the side panel and secure it.

- Reconnect the monitor cable to the new graphics card’s video output (HDMI, DisplayPort, etc.).

- Plug the power cable back in.

8. Power On and Install Drivers

- Turn on the PC.

- If the system boots successfully, install the latest graphics drivers from the manufacturer’s website (NVIDIA, AMD, or Intel).

- Restart the system after installation.

9. Test the Graphics Card

- Check Display Settings or Device Manager to confirm the card is detected.

- Run a game or GPU benchmark to ensure it’s working properly.

- What is RAID, and what are some common RAID configurations?

Ans. RAID (Redundant Array of Independent Disks)
is a technology that combines multiple hard drives into a single system to improve performance, data reliability, or both.

It allows data to be stored across several drives in different ways (called RAID levels), depending on whether the goal is speed, data protection, or storage capacity. In Simple Terms:

- RAID 0 → Speed

- RAID 1 → Safety

- RAID 5 → Speed + Safety

- RAID 6 → Extra Safety

- RAID 10 → Best of Both (but costly)

- Demonstrate how to replace a CPU fan in a desktop computer.

Ans. 1. Power Off and Unplug

- Turn off your computer completely.

- Unplug the power cable and all connected devices.

- Press the power button once after unplugging to discharge remaining electricity.

2. Open the Computer Case

- Remove the side panel of the CPU cabinet using a screwdriver.

- Make sure you are working on a clean, static-free surface.

3. Locate the CPU Fan

- The CPU fan is mounted on top of the CPU heatsink, which is attached to the motherboard’s CPU socket.

4. Disconnect the Fan Cable

- Gently unplug the CPU fan’s power connector from the CPU_FAN header on the motherboard.

5. Remove the Old Fan (and Heatsink if Combined)

- If your fan is attached to a heatsink, unscrew or unclip the heatsink-fan assembly carefully.

- Lift it straight up to avoid damaging the CPU socket.

- If thermal paste is present, gently twist the heatsink before lifting to break the seal.

6. Clean the Surface (if removing heatsink)

- Use a soft cloth or isopropyl alcohol to clean old thermal paste from the top of the CPU and the bottom of the heatsink.

7. Apply New Thermal Paste (if needed)

- Apply a small pea-sized drop of thermal paste in the center of the CPU.

8. Install the New Fan

- Place the new fan or fan-heatsink assembly directly over the CPU socket.

- Align it with the mounting holes or clips.

- Secure it tightly using screws or clips (don’t overtighten).

9. Connect the Fan Cable

- Plug the new fan’s power connector into the CPU_FAN header on the motherboard.

10. Close the Case and Test

- Reattach the side panel and plug the power cable back in.

- Turn on the PC and check if the new CPU fan is spinning and running quietly.

- Monitor CPU temperature to ensure proper cooling.

- Discuss the importance of regular maintenance for computer hardware and provide examples of maintenance tasks.

Ans. Regular maintenance of computer hardware is very important to ensure that the system runs smoothly, efficiently, and lasts longer. It helps prevent hardware failures, improves performance, and reduces the risk of data loss or system downtime.

Importance of Regular Maintenance:

- Increases Lifespan:
Regular cleaning and care prevent dust and heat buildup, which can damage internal parts.

- Prevents System Failures:
Detecting issues early (like overheating or faulty fans) helps avoid sudden breakdowns.

- Improves Performance:
Keeping components clean and drivers updated ensures the system runs faster and more efficiently.

- Reduces Repair Costs:
Preventive maintenance is cheaper than replacing damaged parts later.

- Ensures Data Safety:
Properly functioning hardware reduces chances of data corruption or hard drive crashes.

Examples of Maintenance Tasks:

- Cleaning the Computer:

  - Remove dust from fans, CPU, and keyboard using air blowers or soft brushes.

  - Wipe screen and external parts with a microfiber cloth.

- Checking Connections:

  - Ensure cables and peripherals are properly connected and not damaged.

- Updating Drivers and Firmware:

  - Keep hardware drivers up to date for better compatibility and performance.

- Disk Maintenance:

  - Perform disk cleanup and defragmentation to free up space and improve speed.

- Temperature Control:

  - Ensure proper ventilation and check cooling fans to prevent overheating.

- Power Supply Check:

  - Use a stable power source or UPS to avoid damage from voltage fluctuations.

- Inspecting and Replacing Components:

  - Replace faulty RAM, hard drives, or batteries when they show signs of failure.